EX1:

Schematic:

Source code:

EX2:

Schematic:

Source code:

EX 3:

Schematic:

Source code:

EX4:

Source code:

EX5:

Source code:

Ex6:

Report 1: What if line 1 (setTimer0(1000);) is missing?

The LED never blinks. timer0_counter starts at 0, and timer_run() only counts down when the counter is > 0. So it never counts, never reaches "0 from above", and never sets the flag. main waits forever for a flag that never comes, and setTimer0(2000) inside the if is never reached either.

Report 2: What if it's setTimer0(1)?

The LED never blinks, for the same reason. duration / TIMER_CYCLE = 1 / 10 = 0, because integer division drops the decimal. The counter is set to 0 and the flag is cleared, which is exactly the state from Report 1. Any duration under 10 ms gives 0 ticks, so the timer can't resolve it.

Report 3: What if it's setTimer0(10)? How does it compare to the first two?

10 / 10 = 1 tick. On the next interrupt the counter goes 1 → 0, and the flag is set. So the LED toggles for the first time after about 10 ms, which looks almost immediate. After that, setTimer0(2000) takes over and it toggles every 2 s as usual.

Compared to Reports 1 and 2: 10 ms is the smallest duration that works, because it's exactly one interrupt period. Anything smaller rounds down to 0 ticks and the timer never fires. The software timer's resolution is limited by TIMER_CYCLE.

EX7:

Source code:

EX8:

Source code:

EX9:

Schematic:

Source code:

EX10:

The animation is done by changing the display data. A fourth software timer (timer3) fires every 500 ms, and each time shiftLeftMatrix() moves every column of matrix_buffer one position to the left, wrapping the first column to the end. Meanwhile, updateLEDMatrix() keeps scanning the buffer as in Ex9, so the character appears to scroll continuously to the left. The timer interrupt still only runs timer_run().